# Mathematical Model Formulation: Scale Formation and Inhibitor Dosing in a Pipeline

SCE-701 Directed Research, working draft v0.1

# 0. How to read this draft

Every equation is tagged with where it comes from.

- [Lit]: taken from a paper and checked on the page image or in the extracted text. Source and equation number are given. Paper IDs (2-09, 1-12, …) are the file prefixes in Core_101_Download; the full list is in Model_Equation_Matrix.xlsx.

- [Std]: a standard textbook or geometric relation, not taken from our papers. Please sanity-check these.

- [Prop]: my own modelling proposal (how the pieces are combined). These are the parts to discuss with your supervisor and the senior PhD student.

Sections 1 to 4 describe the physics model. Section 5 is the estimator (EKF/UKF). Section 6 is the inhibitor controller (MPC). Sections 7 to 9 cover parameters, validation and the gaps.

# 1. Scope and assumptions

- System: one pipe segment, length L, inner diameter d0, single-phase water, flow rate Q(t), mean velocity u=4Q/(πd2).

- Scales: BaSO4 is the reference case (most data: 1-07, 1-12, 1-15, 2-09, 5-05). CaCO3 and SrSO4 reuse the same structure with different constants. Fe scales are a later extension.

- One limiting ion pair per scale. For BaSO4 the coupled concentration c is the Ba2+ or SO42- concentration (1 mol of solid removes 1 mol of each; 2-09 Eq. 14).

- 1D plug flow with a wall sink (axial dispersion neglected; radial effects enter only through the mass-transfer coefficient).

- Deposit is thin, adherent and uniform around the circumference; it changes the flow area and the wall thermal resistance.

- Inhibitor is a single sub-stoichiometric scale inhibitor acting by surface adsorption (kink-site Langmuir model, 2-09). 3-16 supports this over a chelation picture.

- Out of scope for v0.1: multiphase flow, flashing and CO2 partitioning, nucleation in the bulk, particle (adhesion) deposition, porous-medium or near-wellbore domains.

# 2. Variables

| Symbol | Meaning | Unit | Role |
| --- | --- | --- | --- |
| c(x,t)​ | coupled limiting-ion concentration | mol/m​3 | state |
| δ(x,t)​ | deposit thickness | m | state (hidden) |
| Cinh(x,t)​ | inhibitor concentration | ppm | state |
| T(x,t)​ | fluid temperature | K | state (optional) |
| Q(t), Tin, cin | flow, inlet temperature, inlet concentration |  | measured inputs |
| Cinh,in(t)​ | injected inhibitor concentration | ppm | control input |
| d(x,t)=d0−2δ​ | effective diameter | m | derived |
| θp​ | parameters (see Section 7) |  | estimated |

# 3. Physics model

## 3.1 Thermodynamics: saturation state

Saturation index [Lit: 2-09, definition in text; 5-05 Eq. 1-2]:

SI=log10IAPKsp,  IAP=γ+γ− [M2+][X2−]

Solubility product as a function of temperature and pressure [Lit: 5-05 Eq. 3]:

lnKsp=A+BT+ClnT+DT+E P−PrefPref,  Pref=0.1 MPa

Activity coefficients γi come from an electrolyte model: eNRTL (5-05 Eq. 4-6) or Pitzer (ScaleSoftPitzer in 2-09 and 1-12; PHREEQC in 2-02 and 2-15).

Equilibrium concentration ceq(T,P,brine) is the concentration at SI=0, computed by the same solver [Lit: 2-09 p.9; 1-12].

Design choice [Prop]. Do not solve the activity model inside the filter. Precompute ceq and SI on a grid of (T,P,composition) with PHREEQC or ASPEN, then interpolate. This keeps the estimator fast and uses ASPEN only for what it does well.

## 3.2 Hydraulics and mass transfer

Coupled-ion diffusivity and its temperature dependence [Lit: 2-09 Eq. 2-3]:

Dcouple=|zazb| DaDb|za|Da+|zb|Db,  Da/b=kBT6πμra/b

Dimensionless groups [Std]:

Re=ρudμ,  Sc=μρDcouple

Sherwood number by flow regime [Lit: 2-09 Eq. 4-9]:

- Laminar, developing, Re<2300 (Hausen): Sh=3.66+0.0668 Gz1+0.04 Gz2/3, with Gz=dlRe Sc

- Turbulent, Re>3100 (Gnielinski): Sh=(f/8)(Re−1000)Sc1+12.7(f/8)1/2(Sc2/3−1), with f=(0.79lnRe−1.64)−2

- Transitional 2300<Re<3100 and rough walls: Churchill-type blend (2-09 Eq. 8-9; copy the exact form from page 6 before coding)

Mass-transfer coefficient [Std]:

km=Sh Dcoupled

Cross-check in the laminar case: Sh=1.62 (Re Sc d/L)1/3 [Lit: 1-12 Eq. 12].

Pressure drop through the narrowing pipe [Std, Darcy-Weisbach]:

∂P∂x=−fD ρu22d

## 3.3 Deposition kinetics (two-phase)

2-09 and 1-15 describe deposition in two phases. Phase 1: an induction time while the first layer forms in the stagnant sublayer. Phase 2: growth limited by diffusion to the wall at a rate set by the flow regime.

Induction time for BaSO4 [Lit: 1-15 Eq. 2; re-check digits in the PDF]. Here cCa is the calcium molality and cBa, cSO4 are the barium and sulfate concentrations:

logtind=1.52−10.87SI−895.7T+5477SI⋅T+0.829 cCa+0.225 logcBacSO4

Lag ramp [Lit: 2-02 Eq. 8, applied here to a local lag clock ts, which is my adaptation, [Prop]]. ts is the time the local SI has stayed above zero:

fd=0ts≤tind1−exp​−(ts−tind)/τts>tind

Surface reaction constant, Arrhenius in temperature [Lit: 2-02 Eq. 7]:

ks=αexp​−ψ1T−1293

Combined rate constant with mass transfer and surface kinetics in series [Std, series resistance; [Prop] as a choice]:

1keff=1km+1ks

In the diffusion-controlled limit keff→km, which is what 2-09 and 1-12 observe in Phase 2.

Inhibitor effect, Langmuir kink-site model [Lit: 2-09 Eq. 11-12]:

ϑ=KeqCinh1+KeqCinh=kr,0−kr,inhkr,0 ⇒ kr,inh=kr,01+KeqCinh

The second form is my algebraic rearrangement; see the check in Section 8. Inhibitor also lengthens the induction time: tind→tind g(Cinh). 2-09 refers to semi-empirical models (Dai et al.); the function g is still missing (Section 9).

Wall flux [Prop, combining the above]:

J(x,t)=fd keff1+KeqCinh (c−ceq)+  (mol m−2s−1)

## 3.4 State equations

Dissolved species, 1D plug flow with wall sink. 4/d is the wall-area-to-volume ratio of a circular tube [Std]. The same structure appears in 2-02 Eq. 11-12:

∂c∂t+u∂c∂x=−4d J

Check against the literature. For steady J=k(c−ceq) and constant d, u, the solution is c(x)=ceq+(c0−ceq)exp(−4kx/(ud)). This is the same as 1-12 Eq. 16 and 2-09 Eq. 13, exp(−kπd x/Q), since Q=uπd2/4. The model reduces to the published tube-test equation.

Deposit growth [Lit: 2-02 Eq. 13 for the form ∂s/∂t=R; molar-volume conversion is Std]:

∂δ∂t=Mwρs J

Inhibitor transport with consumption on the surface [Prop; the consumption term rinh is a gap]:

∂Cinh∂t+u∂Cinh∂x=− rinh(Cinh,J,d)

Energy balance (optional) [Lit: 2-02 Eq. 9-10, 14]. Heat loss to the surroundings through the pipe and scale layers in series; the scale layer adds thermal resistance, which couples δ back to T and then to Ksp and the rate:

ρwCw∂T∂t+u∂T∂x=kw∂2T∂x2+qv(T,Tf,δ)

Boundary and initial conditions [2-02 Eq. 15-16 use the same form]:

c(0,t)=cin(t), Cinh(0,t)=Cinh,in(t), T(0,t)=Tin(t), δ(x,0)=δ0(x), c(x,0)=cinit(x)

# 4. Discretisation

Grid of N nodes along x (upwind finite volume for the advective terms, explicit or semi-implicit in time). State vector:

z=[c1..cN, δ1..δN, Cinh,1..Cinh,N (,T1..TN)]⊤∈R3N (or 4N)

Since u is of order 0.1 to 1 m/s and deposit growth is hours to days, the system is stiff in time scales: c and Cinh respond in seconds to minutes, δ in hours. Use a quasi-steady solution for c at each step (it is algebraic given δ and Cinh), and treat δ and the parameters as the slow states.

# 5. State estimation (EKF/UKF) [Prop, standard filter equations]

Process: zk+1=F(zk,uk;θ)+wk. Measurements yk=H(zk)+vk:

| Measurement | Model H | Source |
| --- | --- | --- |
| Differential pressure | ΔP=0LfDρu2/(2d) dx, d=d0−2δ | [Std]; empirical alternative from 3-07 |
| Effluent ion concentration | c(L,t)​ | 1-12, 2-09 (as in the tube tests) |
| Thickness sensor (ultrasonic, electrical) | δ at sensor nodes | Block F sensing, not yet in this folder |
| Temperature, flow rate | direct |  |
| Residual inhibitor | Cinh at sampling points |  |

Augmented-state variant: append slowly-varying parameters θ=[Keq,α,ψ,τ] to z with random-walk dynamics, so the filter tracks them as scale builds (this is the “adaptive” part). Initial covariance P0 and noise matrices Qw,Rv are tuned on synthetic data first.

# 6. Inhibitor optimisation (MPC) [Prop]

Receding horizon over Np steps, decision variable Cinh,in(t):

minCinh,in k​w1 Cinh,in,k Qk + w2 maxxδk(x)2 + w3 ΔCinh,in,k2

subject to the model of Section 3 and

- minimum inhibitor concentration at the end of the pipe: Cinh(x,t)≥CMIC wherever SI>0

- thickness limit: maxxδ≤δmax (or ΔP≤ΔPmax)

- dosing bounds: 0≤Cinh,in≤Cmax

2-09 states that minimum dosage can be computed for a required protection time or maximum thickness rate, which is the target behaviour; no closed form was in the pages I read.

# 7. Parameters

| Parameter | Value and source | Status |
| --- | --- | --- |
| Ksp constants A to E (BaSO4) | paper 5-05 Table 5 | not yet read |
| γ model (eNRTL or Pitzer) | 5-05, 2-09 | use solver, not hand-coded |
| Dcouple​ | 1.11×10−9 m​2/s at test conditions (2-09 example) | from 2-09 text |
| α, ψ, τ​ | 2.03×10−8 m/s, 9950 K, 16 s (2-02, CaSO4) | placeholder, not BaSO4 |
| Keq (inhibitor) | fit from 2-09 Table 2 (calcite/DTPMP) and 1-07 (BaSO4/PPCA) | to fit (Section 8) |
| CMIC​ | surface vs bulk differ (1-07) | to extract |
| Mw, ρs (BaSO4) | 233.4 g/mol, about 4.5 g/cm​3 | [Std] |
| rinh​ | none yet | gap |

# 8. Validation plan

- Reduce to the tube test. Simulate the model in the constant-coefficient case and reproduce Ceff=Ceq+(C0−Ceq)exp(−krπd l/Q) (2-09 Eq. 13). Pass criterion: exact agreement.

- Mass transfer. Compare km from Section 3.2 with measured kr in 2-09 Table 2 (calcite, 50 to 90 ​∘C) and 1-12 (barite, laminar tubing).

- Inhibitor law. Calcite with DTPMP, 2-09 Table 2, shows kr,0/kr,inh:

| T​ | Cinh (ppm) | kr,0/kr,inh​ | implied Keq=(ratio−1)/C (ppm​−1) |
| --- | --- | --- | --- |
| 70 ​∘C | 0.05 / 0.10 / 0.25 | 2.25 / 3.37 / 6.56 | 25 / 24 / 22 |
| 50 ​∘C | 0.05 / 0.25 | 1.37 / 4.20 | 7.3 / 12.8 |
| 90 ​∘C | 0.05 / 0.10 / 0.25 | 2.33 / 5.07 / 42 | 27 / 41 / 164 |

- This is my own arithmetic from the table values. The simple form kr=kr,0/(1+KeqC) holds well at 70 ​∘C but drifts at 50 and 90 ​∘C, so either Keq depends on temperature, the form differs from the paper’s actual fit, or the 90 ​∘C points are in a different regime. Check this before relying on the dosing law.

- Induction time. Compare Eq. 2 of 1-15 with the experiments in its Table 1.

- Estimator test. Generate synthetic measurements from the model with noise and a hidden δ(x,t); check the EKF/UKF recovers thickness and Keq. Metrics follow Mujahed’s validation list: thickness error, location error, false-alarm rate, decision latency, dosing reduction versus a constant-dose baseline.

# 9. Known gaps and decisions needed

- Inhibitor consumption rinh and the induction-time effect g(Cinh): no equation yet. Look in 2-13 and 3-07, or find the Dai et al. models cited by 2-09.

- CMIC definition: 1-07 reports that the MIC differs between surface and bulk. Decide which one the constraint uses.

- Sensor equations for thickness: need the sensing papers (components 6 and 8, not yet read).

- CaCO3 and SrSO4 constants. Fe scales: 1-01 gives Ksp(T,I) and an Arrhenius rate form to reuse.

- Numerics: stiff time scales (Section 4) need a quasi-steady treatment; to be tested.

- Tool choice (ASPEN vs literature). Suggested split: literature equations for kinetics and deposition (this draft); a solver (PHREEQC or ASPEN eNRTL) only to tabulate ceq and SI; HYSYS, if needed, only for hydraulic boundary conditions. Confirm with your supervisor.

- Unread papers that may add equations: 2-03, 2-04, 2-06, 2-07, 2-08, 2-10, 2-11, 2-12, 5-10, 5-12, and the remaining part of 1-12 and 1-07.

- Equation-copy checks still due: 2-09 Eq. 8-10 exact form, 1-15 Eq. 2 digits.

# 10. Mapping to the research objectives (Mujahed deck)

| Objective | Where in this draft |
| --- | --- |
| Obj 1: multimodal sensor fusion | Section 5 (measurement model H) |
| Obj 2: physics-informed spatio-temporal prediction | Sections 3 and 4 (PDE model, state grid) |
| Obj 3: adaptive inhibitor optimisation | Sections 3.3 and 6 (inhibitor law, MPC with MIC) |